THERMODYNAMIC BASES OF extraction OF ethylene oxide FROM ITS WATER SOLUTION WITH USE SUPERCRITICAL extraction PROCESS

D.D. Achmetlatipova, D.G. Amirchanov, F.M. Gumerov, F.R. Gabitov

KSTU, Russia, 420015, st. K.Marx, 68,Kazan,

The desorption of ethylene oxide from its water solution is one of the most power-intensive processes of chemical technology. The flowchart of traditional technological process catalytic ethylene oxidations and restorings of ethylene oxide includes a catalytic oxidations reactor, absorber of a high pressure, desorber executions distillation and a node of clearing of  ethylene oxide.

One of the most effective approaches to the decision of a power savings problem can become realization of a stage of desorption in the form of sub-and supercritical process with use carbon dioxide as extragent. In this case in extraction column supercritical carbon dioxide and water solution of ethylene oxide in a countercurrent contact, therefore not less than 90 % of ethylene oxide it is extracted in carbon dioxide phase. Traditionally procedure of realization of similar technologies include: experimental research of kinetic’s process and thermodynamic aspects of process (phase equilibrium in system a water solution - ethylene oxide - supercritical extragent); simulation of process as a whole and realization of optimization algorithm; projection of the equipment, its manufacture and introduction in plants.

Results of research of mutual solubility of ethylene oxide and supercritical carbon dioxide; phase distribution of ethylene oxide in system water- ethylene oxide - supercritical carbon dioxide and as kineticses of process of a ethylene oxide’s desorption from its water solution are presented in [1]. Results confirm prospective efficiency of process at its realization at the temperatures near to critical temperature of carbon dioxide, and conform to optimum parameters of realization of process.

THE LITERATURE

[1] F.M.Gumerov, A.N.Sabirzyanov, G.I.Gumerova, A.R. Achunov, F.R.Gabitov. J. Theoretical bases of chemical technologies . №5 (2002) 503-505.